Source Water Protection Plan for Liberty Park Homeowners Association.

IL# 0435600

Westmont, Illinois 60559

# Vision Statement

The Liberty Park Homeowners Association (LPHOA) is committed to providing the necessary resources for protecting our source water. The water supply is viewed as a crucial element to the survival of the community and as such steps have been taken to ensure redundancy and security. Currently a certified operator is employed by the HOA providing oversight in procedures. A camera on the well buildings allows for visual coverage of our two wells providing a layer of security. Financially our water department is established to secure clear funding and budgeted for clear accounting and planning for present and future needs. Over the last 20 years many upgrades have been undertaken including upgrading 2 well pumps and installation of backup power for two wells. One booster pump has been replaced and the storage tanks have been cleaned and re-sealed. There are currently plans in the works to replace the other well pump and undertake several other projects. While these specific examples of system investment will become dated, they are evidence of the community commitment to providing a reliable and ever improving service and will be replaced by newly completed projects.

In agreement with drinking water industry standards a muti-barrier approach to

our source water protection is used. A minimum setback zone of 200 feet from the Liberty Park Homeowners Association (LPHOA) wells is enforced. Both wells are constructed with sound integrity and proper site conditions located in a confined aquifer of sand and gravel overlain by relatively impermeable glacial till that protects from contamination. The water produced from these wells has been tested for numerous contaminants including but not limited to IOCs, VOCs, and SOCs to ensure they are a high-quality source with ongoing testing to ensure they are continuing to provide safe water for the community. Finally, these wells are evaluated and inspected by the Liberty Park Homeowners Association (LPHOA) on a

regular basis to ensure that any necessary maintenance and repairs can be completed proactively to protect the integrity and reliability of the supply.

This vision statement was developed by Joe Ferrell the responsible operator in charge and Steve South one of the water superintendents with input and feedback from the Liberty Park Homeowners Association (LPHOA) board that all attend regularly scheduled meetings.

# Source Water Assessment

Our source water is critical to the survival of the community providing drinking water and fire protection. Currently Liberty Park Homeowners Association (LPHOA) water does not provide water to any other suppliers or have any interconnections. Well #2 has a Well ID number of 20510. This is an active well that is 278 feet deep with a 200-foot setback zone and produces roughly 250 gallons per minute and utilizes a shallow bedrock aquifer overlain by till or other fined grained material of low permeability. Well #3 has a Well ID number of 20511. It is also active with a depth of 278 feet and a 200-foot setback zone while producing around 220 gallons per minute and utilizes a shallow bedrock aquifer overlain by till or other fined grained material of low permeability. These wells are not considered geologically sensitive by the Illinois EPA.

Sampling results for our source and finished water supplies are available through two sources. The first source is copies of Consumer Confidence Reports which can be obtained by contacting the Liberty Park Homeowners Association (LPHOA) Secretary and requesting the annual reports for your review. The second source of information is available through the IEPA Drinking Water Watch webpage. Both sources contain information on when and where the samples were taken and the results as reported by certified laboratories, at this point we use Pace labs.

The results of sampling over the last ten years have proven our source water to be of very good quality. While these results are promising it is even more assuring to know that our source water results have remained very consistent with little change in the overall mineral and chemical detections. This shows a trend for stability.

The stability of our source water has provided a similar trend of consistency in our finished water. In a time of ever-increasing regulation requiring more frequent and extensive testing (e.g. PFAS, UCMR, lead and copper) our finished water has continued to meet or exceed the regulatory requirements for health and safety.

There are several potential sources of contamination within the 1000-foot setback radius of our two wellheads. All distances listed will be in relation to Well #2 (20510) at 4101 N Washington, since it is located closest to the possible sources of contamination. The nearest risk is at 425 and 525 ft are the Downers Grove Sanitary Lift Station, both are monitored and inspected every 6 months for integrity and operation by the village of Downers Grove. At 625 feet is Buttery Rental which does store fuel on site, but it is in small amounts and their tanks are above ground and regularly inspected.

Due to both wells being of sound construction in an aquifer with a hydrogeologic barrier there is little risk of pathogen movement. Sanitary defects have been mitigated and monitoring data does not indicate a history of disease outbreak.

Further sampling has also indicated there is no IOC, SOC, or VOC contamination. This information along with the consistently stable well data and results indicates a low probability of contamination to the source water.

The Liberty Park Homeowners Association (LPHOA) has taken numerous steps to protect the sources of water. A backup power supply has been purchased to operate the wells and treatment plant in the event of power outage. Video surveillance has been installed providing a measure of security for the water plant and wells. Building structures, well equipment, and plant operating equipment have all been upgraded over time to improve operations and sanitary conditions. An active cross connection control program helps to prevent contamination from end users in the system.

In addition, the Liberty Park Homeowners Association (LPHOA) completes ongoing sampling, testing, and employs a certified operator. These measures show the history and desire of the system to protect the source water while ensuring a reliable community supply of high-quality water.

# Source Water Protection Plan Objectives

There are several objectives the Liberty Park Homeowners Association (LPHOA)  strives to achieve. The first of which is meeting the requirements set forth by the U.S. and Illinois Environmental Protection Agency. Second, the community wants to remain forward thinking to plan for the future. Finally, the Liberty Park HOA is committed to actively addressing issues that are presented to the water system.

# Action Plan

Our first objective of meeting the requirements set forth by the U.S. and Illinois Environmental Protection Agency is an ever changing, ongoing, daily operation. This requires collaboration between the water operator staff and the Liberty Park Homeowners Association (LPHOA) Board. The Liberty Park Homeowners Association (LPHOA) completes annual Consumer Confidence Reports, has an active Cross Connection Control Program, completes regular sampling, Lead and Copper Audits, Source Water Protection Plans, Service Line Inventories, and Monthly Operational Reports. Continued oversite of the Liberty Park Homeowners Association (LPHOA) Board and collaboration with the water operator will allow the Liberty Park Homeowners Association (LPHOA) to address the ongoing and any new or future requirements of the previously mentioned administrative agencies as they arise.

The second objective of remaining forward thinking is accomplished through several means. The future needs of the system are considered regularly using logic, general observation and inspection of infrastructure, and information of possible future regulatory requirements. These considerations are used to guide expense and rate planning during the annual review of the water budget. This is evident through the many infrastructure upgrades in just the last decade. The water storage tanks being inspected and repainted, a backup generator purchased, plans to replace the security fencing around the well property, variable frequency drives have been installed on one pump and planned for the other, security cameras monitor the pump house, one well pump has been replaced and another is planned for replacement in the next year. These investments help to secure and protect our source water. The list is too extensive to include everything. These improvements and acceptance of new technology in our operations is ongoing and continuous. This showcases a commitment to being forward thinking in operation and protection of our supply.

Actively addressing issues presented to the water system is ongoing and occurs daily. This objective is achieved regularly by correcting any potential problems as they are identified. Well pumps are replaced using a proactive approach when observations hint at possible issues with flow or operation. Regular sampling results are reviewed to look for changes in water quality that may require action.

Outside of EPA acceptance of this document there are currently no potential problems or obstacles to implementing the plan.